Cisco Packet Tracer 9.3.1.1
Configuring ASA Basic Settings and Firewall Using CLI

Comprehensive Configuration Guide for Cisco ASA 5505 (Inside, Outside, DMZ, PAT, MPF, and SSH)

| Lab Activity File | 9.3.1.1 Packet Tracer - Configuring ASA Basic Settings and Firewall Using CLI.pka |
| --- | --- |
| Target Device | Cisco ASA 5505 Security Appliance (CCNAS-ASA) |
| Key Technologies | Inside/Outside/DMZ Security Levels, Dynamic PAT, MPF ICMP Inspection, AAA SSH, Static NAT |
| Completion Target | 100% Score (Full Assessment Check Verification) |

1. Lab Scenario & Objectives

The Cisco Adaptive Security Appliance (ASA) 5505 delivers enterprise-class stateful packet inspection, network address translation, virtual private networking, and perimeter access control.

In this enterprise scenario, the ASA serves as the edge security perimeter connecting three distinct network segments:

1. Inside Network: The high-security private enterprise LAN (192.168.1.0/24, Security Level 100). The ASA runs a DHCP server to dynamically assign IP parameters to inside hosts.

2. Outside Network: The untrusted public Internet link (209.165.200.224/29, Security Level 0) connected to ISP gateway router R1. Inside hosts reach the Internet via dynamic Port Address Translation (PAT).

3. DMZ Network: An isolated semi-public network (192.168.2.0/24, Security Level 70) hosting a public web server. The web server is translated via static 1:1 NAT (209.165.200.227) and protected by an inbound access-list.

ASSESSMENT SCORING NOTE: This Packet Tracer activity starts with 20% of the assessment items already completed. These represent default ASA factory baseline properties (such as the default interface name 'inside') that must not be altered.

2. Addressing Table & Credentials

| Device | Interface | IP Address | Subnet Mask | Default Gateway | Security / Role |
| --- | --- | --- | --- | --- | --- |
| R1 | G0/0 S0/0/0 (DCE) | 209.165.200.225 10.1.1.1 | 255.255.255.248 255.255.255.252 | N/A N/A | ISP CPE Transit to R2 |
| R2 | S0/0/0 S0/0/1 (DCE) | 10.1.1.2 10.2.2.2 | 255.255.255.252 255.255.255.252 | N/A N/A | Internet Backbone Core Router |
| R3 | G0/1 S0/0/1 | 172.16.3.1 10.2.2.1 | 255.255.255.0 255.255.255.252 | N/A N/A | Remote ISP Gateway Edge Router |
| ASA 5505 | Vlan 1 (E0/1) Vlan 2 (E0/0) Vlan 3 (E0/2) | 192.168.1.1 209.165.200.226 192.168.2.1 | 255.255.255.0 255.255.255.248 255.255.255.0 | N/A N/A N/A | inside (100) outside (0) dmz (70) |
| DMZ Server | NIC | 192.168.2.3 | 255.255.255.0 | 192.168.2.1 | Static NAT: .227 |
| PC-B | NIC | DHCP Client | 255.255.255.0 | 192.168.1.1 | Inside Host |
| PC-C | NIC | 172.16.3.3 | 255.255.255.0 | 172.16.3.1 | Remote Admin Host |

Pre-configured administrative credentials in this activity:

• Enable Secret Password: ciscoenpa55 (configured in Part 2; initial password is blank)

• Console Line Password: ciscoconpa55

• Local Administrative User: Username: admin  |  Password: adminpa55

• Domain Name: ccnasecurity.com

3. Part 2: Basic ASA Settings & Interface Security Levels

Configure the appliance hostname, domain name, enable password, and assign security levels to VLAN 1 (inside) and VLAN 2 (outside):

ciscoasa> enable Password: [Press Enter] ciscoasa# configure terminal ciscoasa(config)# hostname CCNAS-ASA CCNAS-ASA(config)# domain-name ccnasecurity.com CCNAS-ASA(config)# enable password ciscoenpa55 ! VLAN 1: Inside Network (Security Level 100) CCNAS-ASA(config)# interface vlan 1 CCNAS-ASA(config-if)# nameif inside CCNAS-ASA(config-if)# ip address 192.168.1.1 255.255.255.0 CCNAS-ASA(config-if)# security-level 100 CCNAS-ASA(config-if)# exit ! VLAN 2: Outside Network (Security Level 0) CCNAS-ASA(config)# interface vlan 2 CCNAS-ASA(config-if)# nameif outside CCNAS-ASA(config-if)# ip address 209.165.200.226 255.255.255.248 CCNAS-ASA(config-if)# security-level 0 CCNAS-ASA(config-if)# exit

4. Part 3: Static Routing, Address Translation, and Inspection Policy

The ASA requires a default gateway of last resort pointing to R1's G0/0 interface (209.165.200.225). In addition, Port Address Translation (PAT) dynamically translates the inside subnet, and the Modular Policy Framework (MPF) inspects ICMP to permit returning echo replies:

! Default Static Route pointing to ISP gateway CCNAS-ASA(config)# route outside 0.0.0.0 0.0.0.0 209.165.200.225 ! Network Object PAT (Dynamic Overload on Outside Interface) CCNAS-ASA(config)# object network inside-net CCNAS-ASA(config-network-object)# subnet 192.168.1.0 255.255.255.0 CCNAS-ASA(config-network-object)# nat (inside,outside) dynamic interface CCNAS-ASA(config-network-object)# exit ! MPF: Enable ICMP Application Inspection in Global Service Policy CCNAS-ASA(config)# class-map inspection_default CCNAS-ASA(config-cmap)# match default-inspection-traffic CCNAS-ASA(config-cmap)# exit CCNAS-ASA(config)# policy-map global_policy CCNAS-ASA(config-pmap)# class inspection_default CCNAS-ASA(config-pmap-c)# inspect icmp CCNAS-ASA(config-pmap-c)# exit CCNAS-ASA(config)# service-policy global_policy global

5. Part 4: DHCP Server, AAA Authentication, and Secure Remote Access

Configure the ASA's internal DHCP daemon for inside clients, define a local administrator account with AAA authentication for SSH, and set access authorization filters:

! DHCP Server Daemon for Inside Clients CCNAS-ASA(config)# dhcpd address 192.168.1.5-192.168.1.36 inside CCNAS-ASA(config)# dhcpd dns 209.165.201.2 interface inside CCNAS-ASA(config)# dhcpd enable inside ! Local Authentication Database & AAA for SSH CCNAS-ASA(config)# username admin password adminpa55 CCNAS-ASA(config)# aaa authentication ssh console LOCAL ! RSA Key & SSH Permissions CCNAS-ASA(config)# crypto key generate rsa modulus 1024 ! When prompted to replace existing keys, type: no CCNAS-ASA(config)# ssh 192.168.1.0 255.255.255.0 inside CCNAS-ASA(config)# ssh 172.16.3.3 255.255.255.255 outside CCNAS-ASA(config)# ssh timeout 10

6. Part 5: DMZ Interface, Static NAT, and Access Control Lists

Establish VLAN 3 with security level 70, assign physical port E0/2 to VLAN 3, configure 1:1 Static NAT for the DMZ Web Server, and apply the inbound access-list:

! VLAN 3: DMZ Interface (Isolated from VLAN 1) CCNAS-ASA(config)# interface vlan 3 CCNAS-ASA(config-if)# ip address 192.168.2.1 255.255.255.0 CCNAS-ASA(config-if)# no forward interface vlan 1 CCNAS-ASA(config-if)# nameif dmz CCNAS-ASA(config-if)# security-level 70 CCNAS-ASA(config-if)# exit ! Assign Physical Port E0/2 to DMZ VLAN 3 CCNAS-ASA(config)# interface Ethernet0/2 CCNAS-ASA(config-if)# switchport access vlan 3 CCNAS-ASA(config-if)# exit ! Static 1:1 NAT Mapping: Public IP 209.165.200.227 -> Server 192.168.2.3 CCNAS-ASA(config)# object network dmz-server CCNAS-ASA(config-network-object)# host 192.168.2.3 CCNAS-ASA(config-network-object)# nat (dmz,outside) static 209.165.200.227 CCNAS-ASA(config-network-object)# exit ! Access-List: Allow Outside Traffic to Private DMZ Server IP CCNAS-ASA(config)# access-list OUTSIDE-DMZ permit icmp any host 192.168.2.3 CCNAS-ASA(config)# access-list OUTSIDE-DMZ permit tcp any host 192.168.2.3 eq 80 CCNAS-ASA(config)# access-group OUTSIDE-DMZ in interface outside ! Save Configuration to Flash CCNAS-ASA(config)# write memory

ASA ACCESS-LIST TARGET RULE: On Cisco ASA software, the access-list permit statement references the private internal IP address of the server (192.168.2.3) rather than its public NAT address (209.165.200.227). The ASA un-translates incoming destination addresses before evaluating the interface access-list.

7. Client Configuration: PC-B IP Mode

To complete the DHCP assessment requirements:

1. Click PC-B on the Packet Tracer logical topology.

2. Select the Desktop tab, then click IP Configuration.

3. Select the DHCP radio button.

4. Confirm PC-B successfully leases an IP in the range 192.168.1.5 - 192.168.1.36 with Default Gateway 192.168.1.1.

8. Verification & Scoring Checklist

1. Ping Outside from PC-B: From PC-B command prompt, execute 'ping 209.165.200.225'. Packets succeed (0% loss), verifying dynamic PAT and MPF return inspection.

2. SSH Remote Management: From PC-B, run 'ssh -l admin 192.168.1.1'. From PC-C, run 'ssh -l admin 209.165.200.226'. Both connect and authenticate successfully.

3. Assessment Check: Click Check Results in the Packet Tracer Activity window. Completion displays 100%.

9. Master Copy-Paste Block: Cisco ASA 5505

Paste the block below into the ASA CLI tab:

enable conf t hostname CCNAS-ASA domain-name ccnasecurity.com enable password ciscoenpa55 interface vlan 1 nameif inside ip address 192.168.1.1 255.255.255.0 security-level 100 interface vlan 2 nameif outside ip address 209.165.200.226 255.255.255.248 security-level 0 route outside 0.0.0.0 0.0.0.0 209.165.200.225 object network inside-net subnet 192.168.1.0 255.255.255.0 nat (inside,outside) dynamic interface class-map inspection_default match default-inspection-traffic exit policy-map global_policy class inspection_default inspect icmp exit service-policy global_policy global dhcpd address 192.168.1.5-192.168.1.36 inside dhcpd dns 209.165.201.2 interface inside dhcpd enable inside username admin password adminpa55 aaa authentication ssh console LOCAL crypto key generate rsa modulus 1024 no ssh 192.168.1.0 255.255.255.0 inside ssh 172.16.3.3 255.255.255.255 outside ssh timeout 10 interface vlan 3 ip address 192.168.2.1 255.255.255.0 no forward interface vlan 1 nameif dmz security-level 70 interface Ethernet0/2 switchport access vlan 3 object network dmz-server host 192.168.2.3 nat (dmz,outside) static 209.165.200.227 access-list OUTSIDE-DMZ permit icmp any host 192.168.2.3 access-list OUTSIDE-DMZ permit tcp any host 192.168.2.3 eq 80 access-group OUTSIDE-DMZ in interface outside write memory