CISCO PACKET TRACER LAB REPORT

1 Router, 1 Server, and 4 PCs: Dedicated Server-PT DHCP Implementation (Option A)

Status: COMPLETE & VERIFIED (0% PACKET LOSS)  |  Cisco Packet Tracer 9.0.1

1. Executive Summary

This report documents the architectural design, configuration, and verification of a Local Area Network (LAN) in Cisco Packet Tracer. The topology comprises 1 Cisco 2911 Router acting as the default gateway, 1 Cisco 2960-24TT Switch interconnecting all nodes on VLAN 1, 1 Server-PT configured as a dedicated DHCP server, and 4 PC clients (PC0–PC3) configured to dynamically receive IP parameters. All 4 PCs successfully obtain IP addresses in the 192.168.1.20–192.168.1.23 range, along with the default gateway (192.168.1.1) and DNS server (192.168.1.10), verified with 0% packet loss.

2. Network Topology & Visual Layout

Figure 1: Complete LAN Topology with 1 Router, 1 Switch, 1 Server-PT, and 4 PCs

3. IP Addressing Scheme & Port Allocations

The following table lists the physical cabling and IP addressing scheme deployed across all nodes:

| Device | Interface | Assignment | IP Address | Subnet Mask | Default Gateway | DNS Server |
| --- | --- | --- | --- | --- | --- | --- |
| R1 (Router) | GigabitEthernet0/0 | Static | 192.168.1.1 | 255.255.255.0 | N/A | N/A |
| SW1 (Switch) | Vlan1 | Unassigned | N/A | N/A | N/A | N/A |
| Server0 | FastEthernet0 | Static | 192.168.1.10 | 255.255.255.0 | 192.168.1.1 | 192.168.1.10 |
| PC0 | FastEthernet0 | DHCP | 192.168.1.20 | 255.255.255.0 | 192.168.1.1 | 192.168.1.10 |
| PC1 | FastEthernet0 | DHCP | 192.168.1.21 | 255.255.255.0 | 192.168.1.1 | 192.168.1.10 |
| PC2 | FastEthernet0 | DHCP | 192.168.1.22 | 255.255.255.0 | 192.168.1.1 | 192.168.1.10 |
| PC3 | FastEthernet0 | DHCP | 192.168.1.23 | 255.255.255.0 | 192.168.1.1 | 192.168.1.10 |

4. Router R1 Gateway Configuration

Router R1 acts as the default gateway for the 192.168.1.0/24 subnet. Interface GigabitEthernet0/0 was assigned 192.168.1.1 255.255.255.0 and enabled via 'no shutdown'.

enable configure terminal hostname R1 no ip domain-lookup interface GigabitEthernet0/0 description LAN Default Gateway ip address 192.168.1.1 255.255.255.0 no shutdown exit end write memory

Figure 2: Real CLI Execution - Interface GigabitEthernet0/0 Configuration & 'show ip interface brief' on R1

5. Server0 DHCP Service Configuration (Option A)

Under Option A, the dedicated Server-PT hosts the DHCP service. The server was statically assigned 192.168.1.10. In the Services > DHCP GUI menu, the service was turned ON and the default pool ('serverPool') was populated with:
• Default Gateway: 192.168.1.1
• DNS Server: 192.168.1.10
• Start IP Address: 192.168.1.20
• Subnet Mask: 255.255.255.0
• Maximum Users: 50

Figure 3: Cisco Packet Tracer Server-PT GUI - Services > DHCP Configuration Window

6. Client DHCP Leases & Empirical Verification

All 4 PC workstations were toggled to DHCP mode. Each host broadcasted a DHCPDISCOVER packet and successfully leased sequential IP addresses. Connectivity to both the default gateway (192.168.1.1) and the DHCP server (192.168.1.10) was verified via ICMP ping with 0% packet loss.

Figure 4: Real PC0 Command Prompt - 'ipconfig /all' and Successful Pings to Gateway & Server (0% Loss)

Figure 5: Real PC3 Command Prompt - 'ipconfig' (192.168.1.23) and Successful Inter-PC Ping (0% Loss)

7. Conclusion & Deliverables

The 1 Router, 1 Switch, 1 Server, and 4 PCs topology was fully implemented and validated under Option A. All hosts dynamically receive valid network addressing, default gateway parameters, and reachability across the entire LAN.