24rp03297

Meekness Bonheur

CISCO PACKET TRACER LAB REPORT

Site-to-Site IPsec VPN Implementation & Real Command Execution Evidence

1. Executive Summary

This technical report documents the step-by-step implementation and real verification of a Site-to-Site IPsec VPN in Cisco Packet Tracer. All required tasks—IP addressing, default routing, ISP static routing, IKE Phase 1 (ISAKMP Policy 12), IPsec Phase 2 (Transform Set 'jojo'), interesting traffic ACL, and crypto map binding—were executed live on Cisco 2811 routers. High-resolution screenshots of the terminal windows during command execution and verification are provided below.

2. Network Topology

Figure 1: Live Logical Workspace Topology in Cisco Packet Tracer

3. Task 1: IP Addressing & Interface Configuration

Each interface was assigned its designated IP address and enabled with 'no shutdown'. FastEthernet0/0 interfaces connect to local switches, while Serial0/0/0 links provide WAN reachability to the ISP.

| Device | Interface | IP Address | Subnet Mask | Default Gateway | Function |
| --- | --- | --- | --- | --- | --- |
| PC0 | FastEthernet0 | 10.1.1.2 | 255.255.255.0 | 10.1.1.1 | HQ Host |
| HQ (Router0) | FastEthernet0/0 | 10.1.1.1 | 255.255.255.0 | N/A | HQ LAN Gateway |
| HQ (Router0) | Serial0/0/0 | 50.1.1.2 | 255.0.0.0 | N/A | WAN Link to ISP |
| ISP (Router1) | Serial0/0/0 | 50.1.1.1 | 255.0.0.0 | N/A | Facing HQ |
| ISP (Router1) | Serial0/0/1 | 60.1.1.1 | 255.0.0.0 | N/A | Facing Branch |
| Branch (Router2) | Serial0/0/0 | 60.1.1.2 | 255.0.0.0 | N/A | WAN Link to ISP |
| Branch (Router2) | FastEthernet0/0 | 11.1.1.1 | 255.255.255.0 | N/A | Branch LAN Gateway |
| PC1 | FastEthernet0 | 11.1.1.2 | 255.255.255.0 | 11.1.1.1 | Branch Host |

Figure 2: Real CLI Execution - Interface IP Configuration & 'show ip interface brief' on HQ

4. Task 2: Default Routing on HQ & Branch Routers

Both edge routers require a default route (0.0.0.0/0) pointing toward their ISP serial gateway. HQ forwards all internet traffic to 50.1.1.1, and Branch forwards all internet traffic to 60.1.1.1.

HQ(config)# ip route 0.0.0.0 0.0.0.0 50.1.1.1 Branch(config)# ip route 0.0.0.0 0.0.0.0 60.1.1.1

Figure 3: Real CLI Execution - Default Routing & 'show ip route static' on HQ

5. Task 3: Static Routing for HQ & Branch on ISP Router

To simulate provider routing across the public core, ISP Router (Router1) was configured with explicit static routes for enterprise subnets: 10.0.0.0/8 via HQ (50.1.1.2) and 11.0.0.0/8 via Branch (60.1.1.2).

ISP(config)# no ip route 0.0.0.0 0.0.0.0 60.1.1.2 ISP(config)# no ip route 0.0.0.0 0.0.0.0 50.1.1.2 ISP(config)# ip route 10.0.0.0 255.0.0.0 50.1.1.2 ISP(config)# ip route 11.0.0.0 255.0.0.0 60.1.1.2

Figure 4: Real CLI Execution - Static Routes & 'show ip route static' on ISP

6. Task 4: Site-to-Site IPsec VPN Configuration Execution

The VPN was configured on both edge routers across 5 stages:
1. IKE Policy 12: 3DES encryption, MD5 hash, Pre-share authentication, Diffie-Hellman Group 1, Lifetime 3600.
2. ISAKMP Pre-Shared Key: 'jojo123' matching peer public IP.
3. Transform Set: 'jojo' (esp-3des esp-md5-hmac).
4. Interesting Traffic ACL: Access-list 120 matching traffic between 10.0.0.0/8 and 11.0.0.0/8.
5. Crypto Map: 'jojo 12 ipsec-isakmp' applied to Serial0/0/0.

Figure 5: Real CLI Execution - Complete IPsec VPN Configuration on HQ Router

Figure 6: Real CLI Execution - Complete IPsec VPN Configuration on Branch Router

7. Task 5: Live Verification & Proof

Verification was performed across three levels: policy status, active cryptographic security associations, and end-to-end data transmission.

A. IKE Policy & Crypto Map Binding

Figure 7: Real CLI Verification - 'show crypto isakmp policy' and 'show crypto map' on HQ

B. Active Security Associations (IKE SA & IPsec SA)

The output below confirms: (1) ISAKMP SA state is 'QM_IDLE ACTIVE', and (2) IPsec SA shows active and

Figure 8: Real CLI Verification - 'show crypto isakmp sa' (QM_IDLE) & 'show crypto ipsec sa' on HQ

C. End-to-End ICMP Ping Test (PC0 to PC1)

Executed directly from PC0 command line: 4 packets sent, 4 received, 0% packet loss with 2ms average latency.

Figure 9: Real PC Command Prompt - 100% Successful Ping (0% Loss) from PC0 to PC1

8. Verification & Diagnostic Command Reference

| Show / Debug Command | Diagnostic Function |
| --- | --- |
| show crypto map | Displays configured crypto maps, transform sets, peer IPs, and active ACL bindings. |
| show crypto isakmp policy | Displays configured Phase 1 IKE policy parameters (encryption, hash, auth, DH group). |
| show crypto isakmp sa | Displays active IKE security associations. Verifies QM_IDLE state. |
| show crypto ipsec transform-set | Displays configured Phase 2 transform suites. |
| show crypto ipsec sa | Displays established IPsec tunnels, SPI identifiers, and packet encapsulation/decryption counters. |
| debug crypto isakmp | Real-time debugging of Phase 1 IKE exchange negotiations. |
| debug crypto ipsec | Real-time debugging of Phase 2 IPsec tunnel negotiation and data encryption events. |