Cisco Packet Tracer 4.4.1.1
Configuring a Zone-Based Policy Firewall (ZPF)

Complete Step-by-Step Command Guide & Verification Procedures for Edge Router R3

| Lab Activity File | 4.4.1.1 Packet Tracer - Configuring a Zone-Based Policy Firewall (ZPF).pka |
| --- | --- |
| Target Device | Router R3 (Cisco 1941 ISR) - Security Technology Package (securityk9) |
| Firewall Architecture | Cisco ZPF (Zone-Based Policy Firewall) — IN-ZONE to OUT-ZONE |
| Completion Target | 100% Score (Full Assessment Check Verification) |

1. Lab Scenario & Objectives

Zone-Based Policy Firewalls (ZPF) are stateful inspection mechanisms in Cisco IOS software. Unlike legacy context-based access control (CBAC) where inspection rules were tied directly to individual interfaces, ZPF abstracts interfaces into security zones. Policies are defined between zone pairs rather than on interfaces.

In this activity, edge router R3 connects the internal corporate LAN (PC-C / 192.168.3.0/24) to the external internet/ISP infrastructure (R2, R1, and PC-A server). You will configure a unidirectional inspection policy allowing internal hosts in IN-ZONE to initiate connections to external resources in OUT-ZONE while automatically dropping all unsolicited inbound traffic.

IMPORTANT LAB NOTE: Cisco Packet Tracer grades exact naming conventions. Ensure zone names (IN-ZONE, OUT-ZONE), class-map (IN-NET-CLASS-MAP), policy-map (IN-2-OUT-PMAP), zone-pair (IN-2-OUT-ZPAIR), and ACL number (101) match precisely.

2. Addressing Table & Pre-Configured Credentials

| Device | Interface | IP Address | Subnet Mask | Default Gateway | Zone Role |
| --- | --- | --- | --- | --- | --- |
| R1 | G0/1 S0/0/0 (DCE) | 192.168.1.1 10.1.1.1 | 255.255.255.0 255.255.255.252 | N/A N/A | External |
| R2 | S0/0/0 S0/0/1 (DCE) | 10.1.1.2 10.2.2.2 | 255.255.255.252 255.255.255.252 | N/A N/A | External |
| R3 | G0/1 S0/0/1 | 192.168.3.1 10.2.2.1 | 255.255.255.0 255.255.255.252 | N/A N/A | IN-ZONE OUT-ZONE |
| PC-A | NIC | 192.168.1.3 | 255.255.255.0 | 192.168.1.1 | External Server |
| PC-C | NIC | 192.168.3.3 | 255.255.255.0 | 192.168.3.1 | Internal Client |

The routers in this activity are pre-configured with the following credentials:

• Enable Secret Password: ciscoenpa55

• Console Line Password: ciscoconpa55

• VTY Line Password: ciscovtypa55

• Local SSH Account: Username: Admin  |  Password: Adminpa55

3. Part 1: Baseline Network Connectivity Verification

Before implementing firewall filters, verify that end-to-end IP reachability, SSH remote administration, and HTTP web services are functional.

Step 1: Test IP Reachability from PC-A to PC-C

Open PC-A > Desktop > Command Prompt and send 4 ICMP echo requests to PC-C:

PC> ping 192.168.3.3 Pinging 192.168.3.3 with 32 bytes of data: Reply from 192.168.3.3: bytes=32 time=... TTL=125 Reply from 192.168.3.3: bytes=32 time=... TTL=125 Reply from 192.168.3.3: bytes=32 time=... TTL=125 Reply from 192.168.3.3: bytes=32 time=... TTL=125 Ping statistics for 192.168.3.3: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss)

Step 2: Access Router R2 via SSH from PC-C

Open PC-C > Desktop > Command Prompt and establish an SSH session to R2's S0/0/1 interface (10.2.2.2):

PC> ssh -l Admin 10.2.2.2 Open Password: Adminpa55 R2> R2> exit [Connection to 10.2.2.2 closed by foreign host]

Step 3: Access PC-A Web Server from PC-C

Open PC-C > Desktop > Web Browser. Enter the URL: 192.168.1.3 and click Go. Verify the Packet Tracer welcome page is displayed, then close the browser window.

4. Part 2: Security License & Firewall Zones on R3

Cisco 1941 routers require the security technology package (securityk9) to unlock ZPF CLI capabilities.

Step 1: Enable Security Technology Package and Reload R3

Open R3 CLI tab, authenticate with enable secret, activate the license, save, and reload:

R3> enable Password: ciscoenpa55 R3# configure terminal R3(config)# license boot module c1900 technology-package securityk9 PLEASE READ THE FOLLOWING TERMS CAREFULLY. INSTALLING THE LICENSE or LICENSE KEY AND/OR USING THE SOFTWARE. .. ACCEPT (yes/no): yes % Please write memory and reload the device for this change to take place. R3(config)# exit R3# copy running-config startup-config Destination filename [startup-config]? [Enter] Building configuration... [OK] R3# reload Proceed with reload? [confirm] [Enter]

RELOAD NOTICE: Wait approximately 5 to 10 seconds in Packet Tracer for R3 to reboot. Issue 'show version' upon reload to confirm 'securityk9' is now listed under Active Technology Packages.

Step 2 & 3: Create Internal and External Security Zones

After R3 has reloaded, define the two security zones:

R3> enable Password: ciscoenpa55 R3# configure terminal R3(config)# zone security IN-ZONE R3(config-sec-zone)# exit R3(config)# zone security OUT-ZONE R3(config-sec-zone)# exit

5. Part 3: Identify Traffic Using a Class-Map

ZPF uses the Cisco Modular QoS CLI (MQC) syntax. First define traffic of interest using an Access Control List (ACL), then bind that ACL inside an inspect class-map.

Step 1: Create Extended ACL 101 for Internal Traffic

Permit all IP traffic originating from the internal subnet 192.168.3.0/24 heading toward any destination:

R3(config)# access-list 101 permit ip 192.168.3.0 0.0.0.255 any

Step 2: Create Inspect Class-Map Matching ACL 101

Create the class-map named IN-NET-CLASS-MAP with match-all parameter:

R3(config)# class-map type inspect match-all IN-NET-CLASS-MAP R3(config-cmap)# match access-group 101 R3(config-cmap)# exit

6. Part 4: Specify Firewall Policies (Policy-Map)

The policy-map establishes what action the router executes on matched traffic. In ZPF, the three possible actions are:

1. inspect: Stateful packet inspection. The firewall tracks connection state in its state table and dynamically creates temporary pinhole openings for return traffic.

2. pass: Traffic is allowed through without state tracking (stateless). Return traffic must be explicitly permitted.

3. drop: Traffic is silently discarded (or with ICMP unreachable messages).

Step 1, 2, & 3: Configure Policy-Map IN-2-OUT-PMAP

Reference class-map IN-NET-CLASS-MAP and assign the inspect action:

R3(config)# policy-map type inspect IN-2-OUT-PMAP R3(config-pmap)# class type inspect IN-NET-CLASS-MAP R3(config-pmap-c)# inspect %No specific protocol configured in class IN-NET-CLASS-MAP for inspection. All protocols will be inspected. R3(config-pmap-c)# exit R3(config-pmap)# exit

7. Part 5: Apply Firewall Policies to Zone-Pair and Interfaces

To enforce the policy, bind the policy-map to a zone-pair connecting source IN-ZONE to destination OUT-ZONE, and assign R3's interfaces to the respective zones.

Step 1 & 2: Create Zone-Pair IN-2-OUT-ZPAIR and Attach Policy

R3(config)# zone-pair security IN-2-OUT-ZPAIR source IN-ZONE destination OUT-ZONE R3(config-sec-zone-pair)# service-policy type inspect IN-2-OUT-PMAP R3(config-sec-zone-pair)# exit

Step 3: Assign Interfaces to Security Zones

Assign G0/1 (internal LAN) to IN-ZONE and S0/0/1 (WAN link to R2) to OUT-ZONE:

R3(config)# interface GigabitEthernet0/1 R3(config-if)# zone-member security IN-ZONE R3(config-if)# exit R3(config)# interface Serial0/0/1 R3(config-if)# zone-member security OUT-ZONE R3(config-if)# exit

Step 4: Save Configuration

R3(config)# end R3# copy running-config startup-config Destination filename [startup-config]? [Enter] Building configuration... [OK]

8. Part 6: Verify Firewall Functionality (IN-ZONE -> OUT-ZONE)

Verify that internal hosts can still access external services and observe how R3 dynamically creates state table sessions.

Step 1: Ping from PC-C to External PC-A Server

Execute from PC-C Command Prompt:

PC> ping 192.168.1.3 Pinging 192.168.1.3 with 32 bytes of data: Reply from 192.168.1.3: bytes=32 time=... TTL=125 Reply from 192.168.1.3: bytes=32 time=... TTL=125 Ping statistics for 192.168.1.3: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss)

Step 2: SSH from PC-C to R2 & View Sessions on R3

On PC-C, initiate an SSH connection to R2:

PC> ssh -l Admin 10.2.2.2 Password: Adminpa55 R2>

While keeping the SSH session open on PC-C, switch to R3 CLI and inspect active sessions:

R3# show policy-map type inspect zone-pair sessions Zone-pair: IN-2-OUT-ZPAIR Service-policy inspect : IN-2-OUT-PMAP Class-map: IN-NET-CLASS-MAP (match-all) Match: access-group 101 Inspect Established Sessions Session 00000001 (192.168.3.3:1025)=>(10.2.2.2:22) tcp SIS_OPEN

Return to PC-C and enter 'exit' to terminate the SSH session.

Step 3: Web Browser Session from PC-C to PC-A Server

Open PC-C Web Browser and navigate to 192.168.1.3. Once the page displays, immediately check R3 CLI:

R3# show policy-map type inspect zone-pair sessions Zone-pair: IN-2-OUT-ZPAIR Service-policy inspect : IN-2-OUT-PMAP Class-map: IN-NET-CLASS-MAP (match-all) Match: access-group 101 Inspect Established Sessions Session 00000002 (192.168.3.3:1026)=>(192.168.1.3:80) tcp SIS_OPEN

9. Part 7: Verify Firewall Blocking (OUT-ZONE -> IN-ZONE)

Verify that unsolicited inbound traffic originating from external sources is completely blocked and dropped by R3.

Step 1: Attempt Inbound Ping from External PC-A to Internal PC-C

Open PC-A Command Prompt and attempt to ping PC-C:

PC> ping 192.168.3.3 Pinging 192.168.3.3 with 32 bytes of data: Request timed out. Request timed out. Request timed out. Request timed out. Ping statistics for 192.168.3.3: Packets: Sent = 4, Received = 0, Lost = 4 (100% loss)

Step 2: Attempt Inbound Ping from Router R2 to Internal PC-C

Open R2 CLI and send 5 ICMP echo requests to PC-C:

R2# ping 192.168.3.3 Type escape sequence to abort. Sending 5, 100-byte ICMP Echos to 192.168.3.3, timeout is 2 seconds: ..... Success rate is 0 percent (0/5)

SECURITY BEHAVIOR CONFIRMATION: Because there is no zone-pair defined for source OUT-ZONE to destination IN-ZONE, Cisco ZPF applies default-drop behavior to all unsolicited inbound traffic. This confirms the firewall operates as intended.

10. Master Command Reference: Router R3

Below are the exact commands ready to copy and paste into R3 in two stages.

Stage 1: License Activation Block (Run first, then wait for reboot)

enable ciscoenpa55 configure terminal license boot module c1900 technology-package securityk9 yes exit copy running-config startup-config reload

Stage 2: Complete ZPF Configuration Block (Run after reboot)

enable ciscoenpa55 configure terminal zone security IN-ZONE exit zone security OUT-ZONE exit access-list 101 permit ip 192.168.3.0 0.0.0.255 any class-map type inspect match-all IN-NET-CLASS-MAP match access-group 101 exit policy-map type inspect IN-2-OUT-PMAP class type inspect IN-NET-CLASS-MAP inspect exit exit zone-pair security IN-2-OUT-ZPAIR source IN-ZONE destination OUT-ZONE service-policy type inspect IN-2-OUT-PMAP exit interface GigabitEthernet0/1 zone-member security IN-ZONE exit interface Serial0/0/1 zone-member security OUT-ZONE exit end copy running-config startup-config