Cisco Packet Tracer 8.4.1.2
Configure and Verify a Site-to-Site IPsec VPN Using CLI

Comprehensive Step-by-Step Configuration & Verification Guide for Routers R1 and R3

| Lab Activity File | 8.4.1.2 Packet Tracer - Configure and Verify a Site-to-Site IPsec VPN using CLI.pka |
| --- | --- |
| Target Devices | Router R1 (Site 1 / 10.1.1.2) & Router R3 (Site 2 / 10.2.2.2) |
| VPN Protocols | IKE Phase 1 (ISAKMP Policy 10, DH 5, AES 256) & IPsec Phase 2 (ESP-AES, ESP-SHA-HMAC) |
| Completion Target | 100% Score (Verified Assessment Check & Counter Inspection) |

1. Lab Scenario & Objectives

In modern network architectures, sensitive corporate communications traversing public or shared transit networks must be protected against eavesdropping, interception, and tampering. IPsec (IP Security) provides network-layer confidentiality, data integrity, and origin authentication.

In this scenario, edge routers R1 and R3 must be configured to establish a site-to-site IPsec VPN tunnel across intermediate transit router R2. Router R2 functions purely as an IP pass-through router without any VPN termination or cryptographic knowledge.

Traffic between the internal LANs (192.168.1.0/24 on R1 and 192.168.3.0/24 on R3) is designated as 'interesting traffic' and automatically encrypted into ESP packets. In contrast, uninteresting traffic (such as communications directed toward R2's LAN at 192.168.2.0/24) transits normally in cleartext without invoking IPsec encryption.

2. Addressing Table & Pre-Configured Credentials

| Device | Interface | IP Address | Subnet Mask | Default Gateway | Switch Port |
| --- | --- | --- | --- | --- | --- |
| R1 | G0/0 S0/0/0 (DCE) | 192.168.1.1 10.1.1.2 | 255.255.255.0 255.255.255.252 | N/A N/A | S1 F0/1 N/A |
| R2 | G0/0 S0/0/0 S0/0/1 (DCE) | 192.168.2.1 10.1.1.1 10.2.2.1 | 255.255.255.0 255.255.255.252 255.255.255.252 | N/A N/A N/A | S2 F0/2 N/A N/A |
| R3 | G0/0 S0/0/1 | 192.168.3.1 10.2.2.2 | 255.255.255.0 255.255.255.252 | N/A N/A | S3 F0/5 N/A |
| PC-A | NIC | 192.168.1.3 | 255.255.255.0 | 192.168.1.1 | S1 F0/2 |
| PC-B | NIC | 192.168.2.3 | 255.255.255.0 | 192.168.2.1 | S2 F0/1 |
| PC-C | NIC | 192.168.3.3 | 255.255.255.0 | 192.168.3.1 | S3 F0/18 |

Pre-configured administrative credentials in this activity:

• Enable Secret Password: ciscoenpa55

• Console Line Password: ciscoconpa55

• VTY Line Password: ciscovtypa55

• SSH Administrative Account: Username: SSHadmin  |  Password: ciscosshpa55

• Routing Protocol: Process ID 101 (Pre-configured across R1, R2, R3)

3. Cryptographic Parameters Matrix

The table below details both phases of the IPsec negotiation architecture:

| Parameter | Router R1 (Site 1) | Router R3 (Site 2) |
| --- | --- | --- |
| IKE Phase 1 Policy Number | Policy 10 | Policy 10 |
| Phase 1 Encryption & Hash | AES 256 / SHA-1 | AES 256 / SHA-1 |
| Key Exchange & Authentication | DH Group 5 / Pre-Shared Key | DH Group 5 / Pre-Shared Key |
| ISAKMP Pre-Shared Key & Peer | vpnpa55 -> Peer 10.2.2.2 | vpnpa55 -> Peer 10.1.1.2 |
| IPsec Transform-Set (Phase 2) | VPN-SET (esp-aes esp-sha-hmac) | VPN-SET (esp-aes esp-sha-hmac) |
| Interesting Traffic (ACL 110) | 192.168.1.0/24 -> 192.168.3.0/24 | 192.168.3.0/24 -> 192.168.1.0/24 |
| Crypto Map & Bound Interface | VPN-MAP 10 -> Serial 0/0/0 | VPN-MAP 10 -> Serial 0/0/1 |

4. Part 1: Configure IPsec Parameters on Router R1

Step 1: Verify Baseline Connectivity

Before configuring cryptographic tunnels, confirm end-to-end routing from PC-A to PC-C:

PC> ping 192.168.3.3 Pinging 192.168.3.3 with 32 bytes of data: Reply from 192.168.3.3: bytes=32 time=... TTL=125 Reply from 192.168.3.3: bytes=32 time=... TTL=125 Ping statistics for 192.168.3.3: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss)

Step 2: Enable Security Technology Package on R1

Activate securityk9, accept the EULA, save the configuration, and reboot R1:

R1> enable Password: ciscoenpa55 R1# configure terminal R1(config)# license boot module c1900 technology-package securityk9 ACCEPT (yes/no): yes % Please write memory and reload the device for this change to take place. R1(config)# exit R1# copy running-config startup-config Destination filename [startup-config]? [Enter] Building configuration... [OK] R1# reload Proceed with reload? [confirm] [Enter]

Step 3: Define Interesting Traffic (ACL 110) on R1

After rebooting, configure extended ACL 110 to identify packets from R1 LAN to R3 LAN:

R1> enable Password: ciscoenpa55 R1# configure terminal R1(config)# access-list 110 permit ip 192.168.1.0 0.0.0.255 192.168.3.0 0.0.0.255

Step 4: Configure IKE Phase 1 ISAKMP Policy on R1

Configure policy 10 with AES 256, pre-shared key authentication, DH group 5, and the secret key for peer R3 (10.2.2.2):

R1(config)# crypto isakmp policy 10 R1(config-isakmp)# encryption aes 256 R1(config-isakmp)# authentication pre-share R1(config-isakmp)# group 5 R1(config-isakmp)# exit R1(config)# crypto isakmp key vpnpa55 address 10.2.2.2

Step 5 & 6: Configure Transform-Set, Crypto Map, and Bind to Interface

Define the Phase 2 transform-set VPN-SET, assemble crypto map VPN-MAP, and bind to Serial 0/0/0:

R1(config)# crypto ipsec transform-set VPN-SET esp-aes esp-sha-hmac R1(config)# crypto map VPN-MAP 10 ipsec-isakmp R1(config-crypto-map)# description VPN connection to R3 R1(config-crypto-map)# set peer 10.2.2.2 R1(config-crypto-map)# set transform-set VPN-SET R1(config-crypto-map)# match address 110 R1(config-crypto-map)# exit R1(config)# interface Serial0/0/0 R1(config-if)# crypto map VPN-MAP *Oct 25 19:40:00.000: %CRYPTO-6-ISAKMP_ON_OFF: ISAKMP is ON R1(config-if)# end R1# copy running-config startup-config

5. Part 2: Configure IPsec Parameters on Router R3

Step 1: Enable Security Technology Package on R3

Open R3 CLI, enable the security package, and reload:

R3> enable Password: ciscoenpa55 R3# configure terminal R3(config)# license boot module c1900 technology-package securityk9 ACCEPT (yes/no): yes R3(config)# exit R3# copy running-config startup-config R3# reload Proceed with reload? [confirm] [Enter]

Step 2: Define Reciprocating Interesting Traffic (ACL 110) on R3

Permit IP traffic from R3 LAN (192.168.3.0/24) heading to R1 LAN (192.168.1.0/24):

R3> enable Password: ciscoenpa55 R3# configure terminal R3(config)# access-list 110 permit ip 192.168.3.0 0.0.0.255 192.168.1.0 0.0.0.255

Step 3: Configure IKE Phase 1 ISAKMP Policy on R3

Configure policy 10 and the pre-shared key pointing to peer R1 (10.1.1.2):

R3(config)# crypto isakmp policy 10 R3(config-isakmp)# encryption aes 256 R3(config-isakmp)# authentication pre-share R3(config-isakmp)# group 5 R3(config-isakmp)# exit R3(config)# crypto isakmp key vpnpa55 address 10.1.1.2

Step 4 & 5: Configure Phase 2 Policy and Bind to Outgoing Interface

Assemble transform-set VPN-SET, crypto map VPN-MAP, and attach to Serial 0/0/1:

R3(config)# crypto ipsec transform-set VPN-SET esp-aes esp-sha-hmac R3(config)# crypto map VPN-MAP 10 ipsec-isakmp R3(config-crypto-map)# description VPN connection to R1 R3(config-crypto-map)# set peer 10.1.1.2 R3(config-crypto-map)# set transform-set VPN-SET R3(config-crypto-map)# match address 110 R3(config-crypto-map)# exit R3(config)# interface Serial0/0/1 R3(config-if)# crypto map VPN-MAP R3(config-if)# end R3# copy running-config startup-config

6. Part 3: Verify the IPsec VPN Tunnel

Step 1: Verify Tunnel Prior to Interesting Traffic

On R1 CLI, check the IPsec security association status. All packet counters must be zero:

R1# show crypto ipsec sa interface: Serial0/0/0 Crypto map tag: VPN-MAP, local addr 10.1.1.2 local ident: (192.168.1.0/255.255.255.0) remote ident: (192.168.3.0/255.255.255.0) current_peer 10.2.2.2 port 500 #pkts encaps: 0, #pkts encrypt: 0, #pkts digest: 0 #pkts decaps: 0, #pkts decrypt: 0, #pkts verify: 0

Step 2 & 3: Generate Interesting Traffic and Confirm Encryption

On PC-A Command Prompt, ping PC-C (192.168.3.3):

PC> ping 192.168.3.3 Pinging 192.168.3.3 with 32 bytes of data: Reply from 192.168.3.3: bytes=32 time=... TTL=125 Reply from 192.168.3.3: bytes=32 time=... TTL=125 Reply from 192.168.3.3: bytes=32 time=... TTL=125 Reply from 192.168.3.3: bytes=32 time=... TTL=125

Immediately re-issue 'show crypto ipsec sa' on R1:

R1# show crypto ipsec sa #pkts encaps: 4, #pkts encrypt: 4, #pkts digest: 4 #pkts decaps: 4, #pkts decrypt: 4, #pkts verify: 4

Also verify Phase 1 ISAKMP SA status on R1:

R1# show crypto isakmp sa IPv4 Crypto ISAKMP SA dst src state conn-id slot status 10.2.2.2 10.1.1.2 QM_IDLE 1 0 ACTIVE

Step 4 & 5: Verify Uninteresting Traffic Bypass

Ping PC-B (192.168.2.3) from PC-A:

PC> ping 192.168.2.3 Pinging 192.168.2.3 with 32 bytes of data: Reply from 192.168.2.3: bytes=32 time=... TTL=126 Reply from 192.168.2.3: bytes=32 time=... TTL=126

Verify that encapsulation counters did NOT increment on R1:

R1# show crypto ipsec sa ¦ include #pkts encaps #pkts encaps: 4, #pkts encrypt: 4, #pkts digest: 4

TUNNEL BYPASS PROOF: The counter remains at exactly 4 packets. Because traffic to 192.168.2.0/24 does not match ACL 110, it is forwarded as cleartext IP packets without triggering the IPsec tunnel.

Step 6: Completion Check

Click Check Results in the Packet Tracer Activity window to verify 100% completion.

7. Master Copy-Paste Blocks

Router R1: Stage 1 License Activation (Run First)

enable ciscoenpa55 configure terminal license boot module c1900 technology-package securityk9 yes exit copy running-config startup-config reload

Router R1: Stage 2 IPsec Configuration (Run After Reboot)

enable ciscoenpa55 configure terminal access-list 110 permit ip 192.168.1.0 0.0.0.255 192.168.3.0 0.0.0.255 crypto isakmp policy 10 encryption aes 256 authentication pre-share group 5 exit crypto isakmp key vpnpa55 address 10.2.2.2 crypto ipsec transform-set VPN-SET esp-aes esp-sha-hmac crypto map VPN-MAP 10 ipsec-isakmp description VPN connection to R3 set peer 10.2.2.2 set transform-set VPN-SET match address 110 exit interface Serial0/0/0 crypto map VPN-MAP exit end copy running-config startup-config

Router R3: Stage 1 License Activation (Run First)

enable ciscoenpa55 configure terminal license boot module c1900 technology-package securityk9 yes exit copy running-config startup-config reload

Router R3: Stage 2 IPsec Configuration (Run After Reboot)

enable ciscoenpa55 configure terminal access-list 110 permit ip 192.168.3.0 0.0.0.255 192.168.1.0 0.0.0.255 crypto isakmp policy 10 encryption aes 256 authentication pre-share group 5 exit crypto isakmp key vpnpa55 address 10.1.1.2 crypto ipsec transform-set VPN-SET esp-aes esp-sha-hmac crypto map VPN-MAP 10 ipsec-isakmp description VPN connection to R1 set peer 10.1.1.2 set transform-set VPN-SET match address 110 exit interface Serial0/0/1 crypto map VPN-MAP exit end copy running-config startup-config